IAC-26 · D5 · IP · 15 · x111222

Probabilistic Solar Eruption Forecasting

for Radiation Risk Mitigation in Deep Space Missions

Research Implementation Plan

Saatvik Sumanta  ·  Kartikey Prasad  ·  Anshika Ranjan  ·  Austin George Francis

77th International Astronautical Congress, 2026

# Overview

This document outlines the complete implementation plan for the probabilistic spatio-temporal solar eruption forecasting framework described in IAC-26 paper x111222. The plan covers all stages from raw data access through model construction, uncertainty quantification, event classification, evaluation, and operational risk integration.

Data source: NASA SDOML v2 (2010–2020), accessed via AWS S3 in Zarr format. Reference model: Surya-1.0 (NASA-IBM, HuggingFace) used as pretrained backbone — not copied, but integrated via weight transfer and LoRA fine-tuning with a novel probabilistic decoder.

# Step 1 — Dataset Construction

## 1.1  Raw Data Access

The primary data source is the SDOML v2 dataset hosted on AWS S3. Access is completely anonymous — no credentials are required.

- Library: Connect to AWS S3 using

- Connect to AWS S3: s3fs.S3FileSystem(anon=True) — no credentials needed

- Bucket path: gov-nasa-hdrl-data1/contrib/fdl-sdoml/fdl-sdoml-v2/

- Zarr stores: Try sdomlv2_small.zarr (512×512) first, then sdomlv2.zarr (full 4096×4096)

- Open root group: zarr.open_group(store, mode='r') — returns hierarchical group object

- Enumerate year groups: [k for k in root.keys() if k.isdigit()] → ["2010", "2011", … "2020"]

## 1.2  Channel Inventory

Each year group contains 13 channel arrays — 10 from AIA and 3 from HMI.

- AIA channels (10): 94Å, 131Å, 171Å, 193Å, 211Å, 304Å, 335Å, 1600Å, 1700Å, 4500Å — stored as separate Zarr arrays per year, shape (T, H, W)

- HMI channels (3): Bx, By, Bz — vector magnetogram components at same spatial shape

- Spatial resolution: Native 4096×4096; SDOML v2 small provides 512×512 downsampled version

- Key naming: _channel_key() handles short ("94A") vs long ("0094") key variants across store versions

## 1.3  Timestamp Alignment (AIA ↔ HMI)

AIA cadence is 6 minutes; HMI cadence is 12 minutes. These are not natively co-registered in time and must be unified.

- Load timestamp index for each channel from Zarr metadata or companion CSV

- For every HMI timestamp t_hmi, find nearest AIA frame where |t_aia − t_hmi| < 3 min

- Discard any HMI frame with no matching AIA frame within the tolerance window

- Result: Unified timeline at 12-minute cadence where every sample has all 13 channels co-registered to the same timestamp

## 1.4  Sliding Window Construction

Each training sample is a (input window, target window) pair extracted by sliding through the unified timeline.

- Input window: T=8 frames × C=13 channels → shape (8, 13, 256, 256) — covers 84 minutes of history

- Target window: 4 forecast horizons → shape (4, 13, 256, 256) covering t+6h, t+12h, t+24h, t+48h

- Stride: 1 step (12 min) during training; stride=4 during validation to reduce overlap

- Skip any window where more than 2 frames are missing across any channel

## 1.5  Event Label Generation

Binary labels are generated per sliding window by cross-referencing external catalogs.

- Flare labels: Download NOAA GOES X-ray flux (XRSB 1–8Å) at 1-minute cadence from NOAA FTP; flag flare_flag=1 if a flare peak ≥ M1.0 occurs within the forecast horizon

- CME labels: Download DONKI CME catalog from NASA CCMC API; flag cme_flag=1 if CME speed > 500 km/s within horizon

- Attach both flags to every sliding window sample as metadata

## 1.6  Train / Validation / Test Split

Splits are strictly temporal — no random shuffling — to prevent data leakage across Solar Cycle phases.

- Train: Solar Cycle 24 rising + maximum (2010–2014)

- Validation: Solar Cycle 24 declining (2015–2017)

- Test: Solar Cycle 24 minimum + Cycle 25 rising (2018–2020)

# Step 2 — Exploratory Data Analysis

## 2.1  Channel Statistics

- Compute per-channel mean, std, min, max, and percentiles (1st, 5th, 95th, 99th) across a random 500-sample subset

- AIA EUV channels (94Å, 131Å) are heavily right-tailed due to flare spikes; visible channels (4500Å) are near-Gaussian

- HMI Bx/By/Bz distributions should show near-zero mean with fat tails in active regions

## 2.2  Temporal Coverage Map

- Plot a heatmap of frame availability per channel per month (2010–2020)

- Identify known data gaps: SDO safe mode events, calibration windows, eclipse seasons

- Solar Cycle 24 maximum (~2012–2014): highest flare rate; minimum (~2019): fewest events

## 2.3  Spatial Structure

- Visualise full-disk AIA 171Å images at three representative dates: solar maximum, minimum, and a major X-class flare

- Overlay HMI Bz magnetogram to show active region polarity inversion lines

- Visualise coronal hole boundaries using AIA 193Å thresholding

## 2.4  Inter-channel Correlations

- Compute Pearson correlation between disk-integrated intensity of each AIA channel pair

- Expected: 94Å and 131Å (hot plasma) highly correlated; 171Å and 193Å (warm corona) correlated; 304Å (chromosphere) partially decoupled

- Plot correlation matrix as a heatmap — informs the rad_loss physics constraint design

## 2.5  Flare Event Distribution

- Plot flare class distribution (A/B/C/M/X) — expected to be heavily imbalanced (X-class < 0.5% of all samples)

- Plot flare occurrence rate per year against sunspot number — should track Solar Cycle 24 amplitude

- Examine 24h and 48h pre-flare AIA 171Å images to identify common precursor signatures visually

## 2.6  Magnetic Field Statistics

- Plot disk-integrated |B| total magnitude per day (2010–2020) — shows Solar Cycle modulation

- Compute spatial gradient maps |∇Bz| for several active regions — high gradient = high complexity = higher flare risk

- Compare |∇Bz| magnitude between flare-producing vs quiet active regions

# Step 3 — Data Cleaning

## 3.1  Missing Frame Handling

- Flag any frame where more than 5% of pixels are NaN or zero-padded

- Isolated single missing frame within a window: linear interpolation between adjacent frames

- Consecutive missing frames (≥ 2): drop the entire window from the dataset

## 3.2  Cosmic Ray Artifact Removal

- Apply median filter with 3×3 kernel; flag pixels that deviate > 5σ from local median

- Replace flagged pixels with the local median value

## 3.3  Degradation Correction Verification

- SDOML v2 provides pre-applied degradation correction — verify by checking long-term channel intensity trends are flat (no systematic decline over 2010–2020)

- If residual trend detected: apply per-year multiplicative correction derived from each channel's 95th-percentile intensity

## 3.4  HMI Disambiguation Artifacts

- HMI disambiguation can produce 180° polarity flip artifacts appearing as sudden jumps

- Flag frames where disk-integrated |ΔBx| between consecutive frames exceeds 3σ of running std

- Drop flagged frames from windows

## 3.5  Solar Limb Masking

- Pixels beyond 0.98 solar radii are noisy due to line-of-sight projection effects

- Create circular mask at 0.98 R☉ centered on disk center; set all pixels outside mask to zero

- Apply consistently to all AIA and HMI channels

# Step 4 — Data Preprocessing & Normalization

## 4.1  AIA Channel Normalization

- EUV channels span 3 orders of magnitude from quiet sun to X-class flare

- Apply arcsinh stretch: x_norm = arcsinh(x / a) where a = channel_median × 0.01

- Standardize to zero mean, unit variance using training-set statistics only (no validation/test leakage)

## 4.2  HMI Normalization

- Apply tanh clipping: x_norm = tanh(x / B_sat) where B_sat = 1500 G

- Saturates strong active region fields without losing weak field structure

- Standardize using training set mean and std

## 4.3  Surya Scaler Alignment (Models D & F)

- For models that use the Surya pretrained encoder, replace the above normalizations with constants from Surya's scalers.yaml

- Critical: mismatched scalers degrade transfer quality — Surya's encoder expects the exact same input distribution it was pretrained on

## 4.4  Spatial Downsampling

- Downsample from 512×512 to 256×256 using area averaging (not bilinear) to preserve integrated flux

- For HMI, area averaging also reduces the noise floor, improving Bz gradient estimates

# Step 5 — Model Architecture Construction

Model construction is organised by the abstract's stated goals. Each goal has distinct model(s) built to achieve it. All models output a (mean, logvar) tuple representing the Gaussian probabilistic prediction.

## Goal 1 — Spatio-Temporal Active Region Evolution (Abstract: video-transformer or diffusion backbone)

This goal requires models that can ingest multi-channel time sequences and produce plausible future solar state predictions while respecting the physical coupling between AIA and HMI channels.

### Model A — ConvLSTM

- Architecture: Encoder ConvLSTMCell → Decoder ConvLSTMCell → Gaussian head outputting (μ, σ²) per channel per forecast step

- Role: Baseline recurrent model; captures local spatial dynamics across time

### Model B — UNet + TemporalAttention

- Architecture: Flatten T×C → spatial UNet encoder-decoder with skip connections + TemporalAttn block at bottleneck across the time dimension

- Role: Multi-scale spatial feature extraction with temporal attention; stronger than ConvLSTM at capturing long-range spatial structure

### Model C — VideoTransformer

- Architecture: Patch-embed each frame at 16×16, add positional + temporal embeddings, run DividedSpaceTimeAttn blocks, decode with cross-attention forecast queries

- Role: Primary video-transformer backbone described in the abstract; global spatial-temporal attention

## Goal 2 — Surya-Initialized Backbone with Novel Decoder (Abstract: physics-informed constraints)

These models use Surya-1.0's pretrained weights as an encoder initialization — not copying the model, but transferring learned solar representations into a novel probabilistic decoder architecture with physics constraints.

### Model D — SuryaLoRAForecaster

- Architecture: Load surya.366m.v1.pt via hf_hub_download; freeze SpectralBlock + DualStreamAttn encoder; add Lin(128→768) input projection + Lin(768→128) output projection; apply LoRA adapters (rank=8, α=16) to all Linear layers inside frozen blocks; novel probabilistic decoder outputs (μ, σ²) at t+6h/+12h/+24h/+48h

- Training: Phase 1 — freeze backbone, train decoder only; Phase 2 — LoRA + decoder fine-tune

- What makes it different from Surya: 4-horizon probabilistic decoder, physics-informed loss, uncertainty output — none of which exist in Surya

### Model E — DiffusionDecoder

- Architecture: Random encoder + UNet denoiser + DDPM schedule (100 steps); 5 denoising trajectories sampled at inference → ensemble mean/std

- Role: Natively probabilistic — generates diverse plausible future solar states; uncertainty arises from the diffusion sampling process itself

### Model F — SuryaDiffusionProbabilistic (Novel Contribution)

- Architecture: Surya pretrained encoder (frozen + LoRA) → DiffusionDecoder (trainable); combines solar-domain pretraining with diffusion-based ensemble generation

- Role: Primary novel architecture; Surya encoder provides solar-domain representations, diffusion decoder generates calibrated ensemble predictions

## 5.3  Physics-Informed Loss (Applied to All Models)

All models are trained with a composite loss that enforces physical consistency between predictions.

- flux_loss: MSE between predicted and target HMI flux sums — enforces magnetic flux continuity ∂Φ/∂t ≈ 0

- div_loss: Penalizes ∂Bx/∂x + ∂By/∂y ≠ 0 — approximates the solenoidal constraint ∇·B = 0

- rad_loss: MSE between Pearson correlations of adjacent AIA channel pairs in prediction vs target — enforces multi-thermal plasma consistency

- Total loss: λ_nll × NLL + MSE + λ_flux × flux_loss + λ_div × div_loss + λ_rad × rad_loss + λ_event × BCE

# Step 6 — Uncertainty Quantification

This step implements the abstract's stated goal of producing calibrated epistemic and aleatoric spatial uncertainty maps instead of deterministic outputs.

## 6.1  Epistemic Uncertainty (Model Uncertainty)

- MC Dropout: Set model to train() mode at inference time (activates dropout), run n=10 forward passes, compute pixel-wise std across passes

- Deep Ensemble: Train 5 independent model instances with different random seeds; compute std across ensemble member predictions

## 6.2  Aleatoric Uncertainty (Data Uncertainty)

- Read directly from the model's predicted σ² output (the Gaussian head's logvar branch)

- Captures irreducible noise intrinsic to the solar data — trained via NLL loss

## 6.3  Calibration

- Compute Expected Calibration Error (ECE) across the validation set

- Plot reliability diagrams — a well-calibrated model lies on the diagonal

- If ECE > 0.05: apply temperature scaling — divide logits by scalar T learned on the validation set

# Step 7 — Eruption Event Classification

This step implements the abstract's goal of estimating flare probabilities and CME likelihood indicators inferred from disk-based eruption proxies such as filament destabilization and coronal dimming.

## Goal 3 — Eruption Proxy Extraction

### Proxy Extractor A — Filament Destabilization (AIA 304Å)

- Compute running-difference maps on AIA 304Å channel

- Threshold at 3σ above background; flag frames with localized intensity increases > 30% as filament activation events

### Proxy Extractor B — Coronal Dimming (AIA 193Å + 211Å)

- Compute running-difference maps on AIA 193Å and 211Å channels

- Flag localized darkening > 20% in a connected region > 50 pixels as post-CME dimming

## Goal 4 — Flare & CME Probability Classification

### EventClassificationHead

- Architecture: Adaptive average pool to 8×8, flatten, 3-layer MLP → P(flare), P(CME), P(SEP)

- Input: Predicted mean frames from each backbone model

### FlareClassificationHead

- Architecture: Conv1D temporal CNN + optional bi-LSTM on EVE irradiance → separate M-class, X-class, CME probability heads

- Training: BCE loss with class imbalance weighting (X-class weight ≈ 20×, M-class ≈ 5×)

## 7.3  SEP Risk Scoring

- Risk formula: risk = 0.7 × P(X-flare) + 0.2 × P(M-flare) × P(CME) + 0.1 × P(CME)

- Alert thresholds: GREEN if risk < 0.2  ·  YELLOW if 0.2–0.5  ·  RED if > 0.5

# Step 8 — Evaluation

The framework is evaluated across multiple Solar Cycle phases, against NOAA operational forecasts, using the metrics described in the abstract: structural similarity, event-based precision-recall, and probabilistic skill.

## 8.1  Image Quality Metrics

- SSIM and MS-SSIM between predicted and ground truth AIA frames per forecast horizon

- Fractions Skill Score (FSS) at spatial scale 16px; threshold = 90th percentile of ground truth

## 8.2  Event Detection Skill

- Precision, Recall, F1, True Skill Statistic (TSS) for flare and CME binary classification

- ROC-AUC per forecast horizon (+6h, +12h, +24h, +48h)

## 8.3  Probabilistic Skill vs NOAA Baseline

- Brier Score and Brier Skill Score (BSS) relative to NOAA SWPC operational deterministic baseline

- Fetch NOAA daily M/X-class probability forecasts and compare at matching horizons

## 8.4  Stratified Evaluation Across Solar Cycle Phases

- Repeat all metrics split by: Solar Cycle phase, flare class (M vs X), and active region size

- Report SSIM decay from +6h to +48h horizon — quantifies forecast skill degradation over time

- Use stratified temporal cross-validation — no random split — to prevent leakage

# Step 9 — Radiation Risk Integration

The final step operationalises the uncertainty-calibrated forecasts into a radiation risk assessment workflow for deep-space mission safety, directly addressing the abstract's stated application objective.

## 9.1  Crew Alert Thresholds

- Map alert tier (GREEN/YELLOW/RED) to NASA permissible exposure limits from SP-6120

- GREEN: risk < 0.2 — nominal operations proceed

- YELLOW: risk 0.2–0.5 — increased monitoring, EVA preparation review

- RED: risk > 0.5 — abort or postpone EVA, shelter-in-place protocol

## 9.2  Dose Uncertainty Propagation

- Feed spatial uncertainty maps into Monte Carlo SEP transport simulation using OLTARIS or HZETRN lookup tables

- Output: cumulative dose probability distribution over a 72-hour mission window in mGy

## 9.3  Operational Forecast API

- Package the full pipeline as a REST API endpoint

- POST /forecast → returns {flare_prob, CME_prob, uncertainty_map, alert_tier, dose_estimate_mGy}

- Mission planners query at EVA decision points to determine go/no-go for extravehicular activity

End of Implementation Plan